- Vernier scale (游標尺)

http://en.wikipedia.org/wiki/Vernier_scale, From Wikipedia, the free encyclopedia

“Nonius” redirects here. For the Roman grammarian, see Nonius Marcellus.

“Vernier” redirects here. For the village, see Vernier, Switzerland.

For the spacecraft component, see Vernier thruster.

A set of vernier calipers.

A vernier scale lets one read more precisely from an evenly divided straight or circular measurement scale. It is fitted with a sliding secondary scale that is used to indicate where the measurement lies when it is in-between two of the marks on the main scale.

It was invented in its modern form in 1631 by the French mathematician Pierre Vernier (1580–1637). In some languages, this device is called a nonius, which is the Latin name of the Portuguese astronomer and mathematician Pedro Nunes (1492–1578) who invented the principle. Another theory is that this name is from the Latin "nona" meaning "9" and therefore "nonius" means a "ninth" of the main scale.

Verniers are common on sextants used in navigation, scientific instruments and machinists' measuring tools (all sorts, but especially calipers and micrometers) and on theodolites used in surveying.

When a measurement is taken by mechanical means using one of the above mentioned instruments, the measure is read off a finely marked data scale (the "fixed" scale, in the diagram). The measure taken will usually be between two of the smallest gradations on this scale. The indicating scale ("vernier" in the diagram) is used to provide an even finer additional level of precision without resorting to estimation.

An enlarged view of the above calipers shows they have a resolution or precision of 0.02 mm. The reading is 3.58 mm. The 3 mm is read off from the upper (fixed) data scale. The 0.58 mm is obtained from the lower (sliding) indicating scale at the point of closest alignment between the two scales. The superimposed red markings show where the readings are taken.
Note In this photograph, parallax error makes it unclear whether the right value is 0.58 mm or 0.60 mm

- Contents - 1 Construction - 2 Use - 3 Examples - 4 Why a vernier scale works - 5 Vernier Acuity - 6 References - 7 See also - 8 External links

- 1. Construction

The indicating scale is constructed so that when its zero point is coincident with the start of the data scale, its gradations are at a slightly smaller spacing than those on the data scale and so do not coincide with any on the data scale. N gradations of the indicating scale would cover N-1 gradations of the data scale (where N is the number of divisions the maker wishes to show at the finer level).

- 2. Use

http://upload.wikimedia.org/wikipedia/commons/c/c2/Using_the_caliper_new_en.gif

Animation of a caliper measurement using a vernier scale. Click to enlarge

When a length is measured the zero point on the indicating scale is the actual point of measurement, however this is likely to be between two data scale points. The indicator scale measurement which corresponds to the best-aligned pair of indicator and data gradations yields the value of the finer additional precision digit.

- 3. Examples

On instruments using decimal measure, as shown in the diagram below, the indicating scale would have 10 gradations covering the same length as 9 on the data scale. Note that the vernier's 10th gradation is omitted.

On an instrument providing angular measure, the data scale could be in half-degrees with an indicator scale providing 30 1-minute gradations (spanning 29 of the half-degree gradations).

- 4. Why a vernier scale works

The vernier scale is constructed so that it is spaced at a constant fraction of the fixed main scale. So for a decimal measuring device each mark on the vernier would be spaced nine tenths of those on the main scale. If you put the two scales together with zero points aligned then the first mark on the vernier scale will be one tenth short of the first main scale mark, the second two tenths short and so on up to the ninth mark which would be misaligned by nine tenths. Only when a full ten marks have been counted would there be an alignment because the tenth mark would be ten tenths, that is a whole main scale unit, short and will therefore align with the ninth mark on the main scale.

Now if you move the vernier by a small amount, say, one tenth of its fixed main scale, the only pair of marks which come into alignment will be the first pair since these were the only ones originally misaligned by one tenth. If we had moved it 2 tenths then the second pair and only the second would be in alignment since these are the only ones which were originally misaligned by that amount. If we had moved it 5 tenths then the fifth pair and only the fifth would be in alignment. And so on for any movement, only one pair of marks will be in alignment and that pair will show what is the value of the small displacement.

- 5. Vernier Acuity

Vernier acuity is the ability to detect the proper alignment of two line segments.[1] In most persons, Vernier acuity is particularly acute. This allows one to differentiate the aligned and misaligned marks on a Vernier scale.

- 6. References

- ^ Vernier acuity definition at the Online Medical Dictionary

- The VERNIER Scale Open Door Web Site reference source for both students and teachers.

- How to use the vernier caliper Clemson University Physics On-line Laboratories

- 7. See also

- Micrometer (device)

- Transversal (instrument making) - technique in use prior to vernier scales.

- 8. External links

- Animated Vernier calipers

| Metalworking |  |
| --- | --- |
| Metalworking measuring instruments | Bore gauge • Caliper • Dial indicator • Engineer's blue • Feeler • Fishtail-Center • Gauge blocks • Gauge • Go-NoGo • Machinist square • Marking blue • Marking gauge • Marking out • Micrometer • Radius gauge • Ruler • Sine bar • Spirit level • Straightedge • Surface plate • Tape measure • Thread pitch • Vernier height • Vernier scale • Wiggler |
| Metalworking topics | Casting • CNC • Cutting tools • Drilling and threading • Fabrication • Finishing • Grinding • Jewellery • Lathe • Machining • Machine tooling • Measuring • Metalworking • Hand tools • Metallurgy • Milling • Occupations • Press tools • Smithing • Terminology • Welding • |

Retrieved from "http://en.wikipedia.org/wiki/Vernier_scale"

Categories: Dimensional instruments | Metalworking measuring instruments | Navigational equipment

- Licensing

I, the copyright holder of this work, hereby publish it under the following licenses: Permission is granted to copy, distribute and/or modify this document under the terms of the GNU Free Documentation License, Version 1.2 or any later version published by the Free Software Foundation; with no Invariant Sections, no Front-Cover Texts, and no Back-Cover Texts. A copy of the license is included in the section entitled "GNU Free Documentation License"..

任何人都可以在自由軟體基金會所公開發行之GNU自由文件協議證書（GFDL）1.2或者之後的版本的授權下，複製、發行、及／或修改此文件；不附帶恆常章節、封面及封底文字等其他附帶條件。

本授權之副本已包含於GNU自由文件協議證書（GFDL）中。